ARAS

Autonomous Risk Assessment System

Full Component List & Technical Reference

Personal Safety Wearable — ESP32 Based IoT Project

# 1. Main Hardware Components

All components used from Phase 1 through Phase 6. Total estimated cost: $25–45 USD depending on supplier.

| # | Component | Function | Qty | Key Specifications | Est. Cost |
| --- | --- | --- | --- | --- | --- |
| 1 | ESP32 Dev Module (30-pin) | Main microcontroller | 1 | WiFi + BT, dual core, 240MHz | ~$4–6 |
| 2 | MPU6050 | Motion + fall detection | 1 | I2C, 0x68, ±8g accel, ±500°/s gyro | ~$1–2 |
| 3 | MAX30102 | Heart rate + SpO2 | 1 | I2C, 0x57, red + IR LED, finger sensor | ~$3–5 |
| 4 | NEO-6M GPS module | Location + speed tracking | 1 | UART 9600 baud, ceramic antenna | ~$4–7 |
| 5 | SSD1306 OLED 0.96" | Live data display | 1 | I2C, 0x3C, 128×64px, white on black | ~$2–3 |
| 6 | ESP32-CAM (AI Thinker) | Evidence capture | 1 | OV2640 camera, microSD slot, flash LED | ~$6–8 |
| 7 | ESP32-CAM-MB board | CAM programmer + USB | 1 | Micro USB port, auto-reset circuit | ~$3–4 |
| 8 | Active buzzer module | Audio alert output | 1 | GPIO 5, 3.3V–5V, 85dB | ~$0.50 |
| 9 | Tactile push button | Manual panic trigger | 1 | GPIO 4, 4-pin, 6×6mm | ~$0.10 |

# 2. Passive & Supporting Components

Breadboard, wires, and accessories needed for prototyping.

| # | Item | Purpose | Qty | Notes |
| --- | --- | --- | --- | --- |
| 1 | Breadboard (830 tie-points) | Prototyping base | 1 | Full size, power rails on both sides |
| 2 | Jumper wires M-M | Component connections | 40+ | Mixed colors: red, black, blue, yellow, green, orange |
| 3 | Jumper wires M-F | Sensor to breadboard | 10+ | For modules with header pins |
| 4 | USB cable (Micro USB) | Programming + power | 2 | One for ESP32, one for ESP32-CAM-MB |
| 5 | MicroSD card | Image storage (Phase 7) | 1 | 8GB–32GB, Class 10 |
| 6 | LiPo battery 3.7V 2000mAh | Portable power (Phase 7) | 1 | For wearable deployment |
| 7 | TP4056 charging module | Battery management (Phase 7) | 1 | Micro USB, overcharge protection |

# 3. Complete Pin Mapping

Every wire connection from component to ESP32 GPIO pin.

| Component | Component Pin | ESP32 Pin | Protocol | Notes |
| --- | --- | --- | --- | --- |
| MPU6050 | VCC | 3.3V | I2C | Power |
| MPU6050 | GND | GND | I2C | Ground |
| MPU6050 | SDA | GPIO 21 | I2C | Data — shared bus |
| MPU6050 | SCL | GPIO 22 | I2C | Clock — shared bus |
| MPU6050 | AD0 | GND | I2C | Sets address to 0x68 |
| MAX30102 | VIN | 3.3V | I2C | Power |
| MAX30102 | GND | GND | I2C | Ground |
| MAX30102 | SDA | GPIO 21 | I2C | Data — shared bus |
| MAX30102 | SCL | GPIO 22 | I2C | Clock — shared bus |
| OLED SSD1306 | VCC | 3.3V | I2C | Power |
| OLED SSD1306 | GND | GND | I2C | Ground |
| OLED SSD1306 | SDA | GPIO 21 | I2C | Data — shared bus |
| OLED SSD1306 | SCL | GPIO 22 | I2C | Clock — shared bus |
| NEO-6M GPS | VCC | 3.3V | UART | Power |
| NEO-6M GPS | GND | GND | UART | Ground |
| NEO-6M GPS | TX | GPIO 16 (RX2) | UART | GPS transmit → ESP receive |
| NEO-6M GPS | RX | GPIO 17 (TX2) | UART | GPS receive ← ESP transmit |
| Panic Button | PIN 1 | GPIO 4 | GPIO | Signal — INPUT_PULLUP |
| Panic Button | PIN 2 | GND | GPIO | Ground |
| Active Buzzer | + (positive) | GPIO 5 | GPIO | Signal — OUTPUT |
| Active Buzzer | - (negative) | GND | GPIO | Ground |

# 4. I2C Device Addresses

All three I2C devices share GPIO 21 (SDA) and GPIO 22 (SCL). Each has a unique address.

| Component | I2C Address | Address Notes | Role |
| --- | --- | --- | --- |
| MPU6050 | 0x68 | AD0 pin connected to GND | Accelerometer + Gyroscope |
| SSD1306 OLED | 0x3C | Default address (some boards: 0x3D) | 128×64 display |
| MAX30102 | 0x57 | Fixed address, cannot change | Heart rate + SpO2 |

# 5. Arduino Libraries Required

Install all libraries via Arduino IDE — Sketch → Include Library → Manage Libraries.

| Library Name | Purpose | Version | Author | Install Via |
| --- | --- | --- | --- | --- |
| Adafruit MPU6050 | MPU6050 sensor driver | v2.2.4+ | Adafruit | Library Manager |
| Adafruit Unified Sensor | Sensor abstraction layer | v1.1.9+ | Adafruit | Library Manager |
| Adafruit SSD1306 | OLED display driver | v2.5.7+ | Adafruit | Library Manager |
| Adafruit GFX Library | Graphics primitives for OLED | v1.11.9+ | Adafruit | Library Manager |
| TinyGPS++ | GPS NMEA sentence parser | v1.0.3 | Mikal Hart | Library Manager |
| SparkFun MAX3010x | Heart rate + SpO2 driver | v1.1.2+ | SparkFun | Library Manager |
| ESP32 Board Package | ESP32 Arduino core | v3.x | Espressif | Board Manager |

# 6. Risk Score Breakdown

Maximum possible score is 100. Score must stay >= 70 for 5 continuous seconds (confidence window) before alert fires.

| Trigger Condition | Points | Description |
| --- | --- | --- |
| MPU6050 — STILL | 0 pts | Normal resting state |
| MPU6050 — MOVING | 15 pts | Active movement detected |
| MPU6050 — FREE-FALL | 30 pts | Near-zero acceleration |
| MPU6050 — IMPACT | 40 pts | Sudden high-g spike after fall |
| Gyro spike bonus | +5 pts | Rapid rotation during movement (>3 rad/s) |
| Heart rate critical high (>130 BPM) | 20 pts | Dangerously elevated HR |
| Heart rate critical low (<40 BPM) | 20 pts | Dangerously low HR |
| Heart rate high (>110 BPM) | 12 pts | Elevated HR |
| Heart rate low (<50 BPM) | 15 pts | Low HR |
| SpO2 critical (<90%) | 20 pts | Critical oxygen level |
| SpO2 low (<94%) | 10 pts | Below normal oxygen |
| GPS sudden stop | 10 pts | Speed >20 km/h drops to <5 km/h |
| Pattern bonus — fall + abnormal vitals | +15 pts | Combined event is more dangerous |
| Panic button pressed | 95 pts (override) | Manual emergency trigger — bypasses confirm window |

# 7. Build Phase Summary

Phase 1 — ESP32 + MPU6050: Motion and fall detection. I2C communication. Serial monitor verification.

Phase 2 — OLED Display added: Live sensor data on screen. SSD1306 joins I2C bus at 0x3C.

Phase 3 — NEO-6M GPS added: UART communication on GPIO 16/17. Location and speed data.

Phase 4 — MAX30102 + Button + Buzzer: Heart rate, SpO2, manual trigger, audio alert. 3-page OLED display.

Phase 5 — ESP32-CAM: Evidence capture on risk trigger. ESP32-CAM-MB for easy programming.

Phase 6 — Risk Score Engine: Full sensor fusion. Weighted scoring. Confidence window. Cancel window.

Phase 7 — Full Integration (pending): ESP-NOW CAM trigger. GSM SMS alerts. Cloud dashboard. Final testing.

ARAS — Autonomous Risk Assessment System  |  ESP32 IoT Safety Project  |  Phase 6 Complete